1. Power losses per harmonic

B_peak(fi) = mu0 * N * I_L1_RMS(fi) / (l_gap + l_core/mu_r)

P_core(fi) = k * fi^alpha * B_peak(fi)^beta * Volume

P_copper   = SUM [ I_L1_RMS(fi)^2 * R_dc ]

P_total    = P_copper + SUM [ P_core(fi) ]

2. Hotspot temperature

T_L1 = T_amb + P_total * Rth_L1

3. Safety check

T_L1 <= 428 K  (155°C Class F insulation, IEC 60085)

4. Lifetime  (Arrhenius — IEC 60085)

L = L_ref * exp( B * (1/T_ref - 1/T_L1) )

L_ref = 20,000 hours

T_ref = 428 K  (Class F)

B     = 7000-9000 K

L_years = L / (365 * 24)

5. Miner's rule

D = SUM [ dt / L_i ]     D >= 1 means end of life

Losses

Let me map out every single input required for the complete inductor pipeline (losses → temperature → lifetime):

Group 1: Core Material Parameters

From the core material datasheet (e.g. Metglas, Magnetics Inc.)

| Parameter | Symbol | Unit | Where to find |
| --- | --- | --- | --- |
| Steinmetz coefficient | k | mW/cm³ | Material datasheet |
| Frequency exponent | a | - | Material datasheet |
| Flux density exponent | b | - | Material datasheet |
| Relative permeability | μr | - | Material datasheet |

Group 2: Core Geometry Parameters

From the specific core part number datasheet

| Parameter | Symbol | Unit | Where to find |
| --- | --- | --- | --- |
| Magnetic path length | ℓ | m | Core part datasheet |
| Cross sectional area | Ae | m² | Core part datasheet |
| Air gap length | ℓg | m | Your design choice |

Group 3: Winding Parameters

You decide these when building the inductor

| Parameter | Symbol | Unit | How to get |
| --- | --- | --- | --- |
| Number of turns | N | - | Design calculation |
| DC resistance | Rdc | Ω | Calculate from wire |
| Wire type | - | - | Your choice (litz, solid) |

Group 4: Thermal Parameters

From thermal datasheet or estimation

| Parameter | Symbol | Unit | Where to find |
| --- | --- | --- | --- |
| Thermal resistance core to ambient | Rth | K/W | Datasheet or estimate |
| Ambient temperature | Tamb | K | Already in your system |

Group 5: Lifetime Parameters

From material aging datasheet or standards

| Parameter | Symbol | Unit | Where to find |
| --- | --- | --- | --- |
| Activation energy | Ea | eV | Material datasheet |
| Reference lifetime | L0 | hours | Datasheet or standard |
| Reference temperature | T0 | K | Datasheet |

Group 6: Operating Inputs

Already computed in your existing pipeline

| Parameter | Symbol | Unit | Where it comes from |
| --- | --- | --- | --- |
| Inductor current waveform | I_L1(t) | A | Your LCL simulation |
| Harmonic frequencies | fj | Hz | FFT of I_L1 |
| Harmonic amplitudes | Ij | A | FFT of I_L1 |